# Title:

ACPI TLBI Domain table

# Status:

Draft

# Document:

ACPI Specification Version 6.next

# License:

SPDX-License-Identifier: CC-BY-4.0

# Submitters:

- Arm

- TianoCore Community (https://www.tianocore.org)

# Summary of the change

The Arm Architecture defines the FEAT_TLBID extension, allowing targeted TLB invalidations. 
This change introduces the TLBI Domain ACPI table, which describes the TLBI domain topology, and also introduces the TLBI Domain Hierarchy Level ACPI table, which describes the “hierarchy level” of each defined TLBI domain.

# Benefits of the change

OSPM will discover the TLBI Domain structure and thus be able to issue TLB invalidations more precisely.

# Impact of the change

Firmware, of platforms implementing Arm FEAT_TLBID, can present these ACPI TLBI tables to describe the TLBI Domain hierarchy to OSPM.

If OSPM performs TLBI domain invalidations, it must be altered to interpret the TLBI ACPI tables.

# Detailed description of the change [normative updates]

- Strikethrough is for removal of existing content.

- Yellow is for re-formatting of existing content.

- Cyan is for new content.

- Pink is for cross-references

…

5.2. ACPI System Description Tables

…

5.2.6. System Description Table HeaderÁ

…

| Table 5.5 DESCRIPTION_HEADER Signatures for tables defined by ACPIÁ |  |  |
| --- | --- | --- |
| Signature | Description | Reference |
| “APIC” | Multiple APIC Description Table | Section 5.2.12 |

…

| “SSDT” | Secondary System Description Table | Section 5.2.11.2 |
| --- | --- | --- |
| “SVKL” | Storage Volume Key Data table in the Intel Trusted Domain Extensions. See Links to ACPI-Related Documents under the heading “Storage Volume Key Data”. |  |
| “TLBI” | TLB Invalidate Domain Table | Section 5.2.38 |
| “TLBH” | TLB Invalidate Domain Hierarchy Level Table | Section 5.2.39 |
| “XSDT” | Extended System Description Table | Section 5.2.8 |

5.2.38. Translation Lookaside Buffer Invalidate Domain Table

This section describes the Arm Translation Lookaside Buffer Invalidate Domain (TLBI) Table.

The Arm Architecture defines the FEAT_TLBID extension, which when present, allows the OSPM to target TLB invalidations to a specific domain.
A TLBI domain can be contained in multiple parent domains.

The TLBI table lists all the PE and SMMU components and the TLBI Domains that each component belongs to.

| Table 5.W Translation Lookaside Buffer Invalidate Domain Table (TLBI) Format |  |  |  |
| --- | --- | --- | --- |
| Field | Byte Length | Byte Offset | Description |
| Signature | 4 | 0 | “TLBI” Signature for the TLBI Domain Table. |
| Length | 4 | 4 | The length of the table, in bytes, of the entire TLBI. |
| Revision | 1 | 8 | 1 |
| Checksum | 1 | 9 | The entire table, including the checksum field, must add to zero. |
| OEMID | 6 | 10 | An OEM-supplied string that identifies the OEM. |
| OEM Table ID | 8 | 16 | An OEM-supplied string that the OEM uses to identify this particular data table. |
| OEM Revision | 4 | 24 | An OEM-supplied revision number. |
| Creator ID | 4 | 28 | The Vendor ID of the utility that created this table. |
| Creator Revision | 4 | 32 | The revision of the utility that created this table. |
| Number of components (M) | 4 | 36 | The number of TLBI Domain components in the Component list. |
| Component list | -- | 40 | List of M TLBI Domain component sub-tables. The TLBI Domain component sub-table is defined in Table 5.X |

Table 5.X  TLBI Domain Component sub-table

| Field | Byte Length | Byte Offset | Description |
| --- | --- | --- | --- |
| Type | 2 | 0 | Component type: 1: PE 2: SMMU All other values are reserved. |
| Reserved | 2 | 2 |  |
| Length | 4 | 4 | Length of this sub-table in bytes, including the Parent domain array. |
| Identifier | 4 | 8 | The identifier of the associated component: If Component type==1: the field contains the “ACPI Processor UID” of the corresponding GICC MADT entry. If Component type==2: the field contains the Identifier number of the corresponding SMMU entry in the IORT. |
| Number of Domains (N) | 4 | 12 | Number of entries in the Domain array.1 |
| Domain array offset | 4 | 16 | Offset in bytes, from the start of the TLBI table to the start of the Domain array. |
| Domain array[N] | -- | 20 | Array of TLBI Domain IDs, with N entries. This component is contained within every domain ID listed in the array. Each Domain ID is represented by a 32-bit integer. |

5.2.39. Translation Lookaside Buffer Invalidate Domain Hierarchy Level Table

This section describes the Arm Translation Lookaside Buffer Invalidate Domain Hierarchy Level (TLBH) Table.

A TLBI domain corresponds to at least a group of PEs, and at most every socket in the system. To aide in synchronising invalidations, a hierarchy level is defined, and each TLBI domain has an assigned level.

The TLBH table lists all TLBI Domains nd the hierarchy level of each domain.

| Table 5.Y Translation Lookaside Buffer Invalidate Domain Hierarchy Level Table (TLBH) Format |  |  |  |
| --- | --- | --- | --- |
| Field | Byte Length | Byte Offset | Description |
| Signature | 4 | 0 | “TLBH” Signature for the TLBI Domain Hierarchy Level Table. |
| Length | 4 | 4 | The length of the table, in bytes, of the entire TLBH. |
| Revision | 1 | 8 | 1 |
| Checksum | 1 | 9 | The entire table, including the checksum field, must add to zero. |
| OEMID | 6 | 10 | An OEM-supplied string that identifies the OEM. |
| OEM Table ID | 8 | 16 | An OEM-supplied string that the OEM uses to identify this particular data table. |
| OEM Revision | 4 | 24 | An OEM-supplied revision number. |
| Creator ID | 4 | 28 | The Vendor ID of the utility that created this table. |
| Creator Revision | 4 | 32 | The revision of the utility that created this table. |
| Number of mappings (M) | 4 | 36 | The number of TLBI Domain Hierarchy Level Mappings in the Mapping list. |
| Mapping list | -- | 40 | List of M TLBI Domain Hierarchy Level Mapping sub-tables. The TLBI Domain Hierarchy Level Mapping sub-table is defined in Table 5.Z |

Table 5.Z  TLBI Domain Hierarchy Level Mapping sub-table

| Field | Byte Length | Byte Offset | Description |
| --- | --- | --- | --- |
| TLBI Domain | 4 | 0 | TLBI Domain Identifier |
| Hierarchy Level | 4 | 4 | TLBI Domain Hierarchy Level. Currently defined levels, and their meanings, are as follows: 0. Root 1. Socket Pair 2. Socket 3. Chiplet Quad 4. Chiplet Pair 5. Chiplet 6. Leaf Pair 7. Leaf NOTE: A “Leaf” is a block of up to 32 PEs, and the finest granularity that a TLBI can be directed to |