# Title:

ACPI TLBI Domain table

# Status:

Draft

# Document:

ACPI Specification Version 6.next

# License:

SPDX-License-Identifier: CC-BY-4.0

# Submitters:

- Arm

- TianoCore Community (https://www.tianocore.org)

# Summary of the change

The Arm Architecture defines the FEAT_TLBID extension, allowing targeted TLB invalidations. 
This change introduces the TLBI Domain ACPI table, which describes the TLBI domain topology.

# Benefits of the change

OSPM will discover the TLBI Domain structure and thus be able to issue TLB invalidations more precisely.

# Impact of the change

Firmware, of platforms implementing Arm FEAT_TLBID, can present this ACPI TLBI table to describe the TLBI Domain hierarchy to OSPM.

If OSPM performs TLBI domain invalidations, it must be altered to interpret the TLBI ACPI table.

# Detailed description of the change [normative updates]

- Strikethrough is for removal of existing content.

- Yellow is for re-formatting of existing content.

- Cyan is for new content.

- Pink is for cross-references

…

5.2. ACPI System Description Tables

…

5.2.6. System Description Table HeaderÁ

…

| Table 5.5 DESCRIPTION_HEADER Signatures for tables defined by ACPIÁ |  |  |
| --- | --- | --- |
| Signature | Description | Reference |
| “APIC” | Multiple APIC Description Table | Section 5.2.12 |

…

| “SSDT” | Secondary System Description Table | Section 5.2.11.2 |
| --- | --- | --- |
| “SVKL” | Storage Volume Key Data table in the Intel Trusted Domain Extensions. See Links to ACPI-Related Documents under the heading “Storage Volume Key Data”. |  |
| “TLBI” | TLB Invalidate Domain Table | Section 5.2.38 |
| “XSDT” | Extended System Description Table | Section 5.2.8 |

5.2.38. Translation Lookaside Buffer Invalidate Domain Table

This section describes the Arm Translation Lookaside Buffer Invalidate Domain (TLBI) Table.

The Arm Architecture defines the FEAT_TLBID extension, which when present, allows the OSPM to target TLB invalidations to a specific domain.
A TLBI domain can be contained in multiple parent domains.

The TLBI table lists all the PE and SMMU components and the TLBI Domains that each component belongs to.

| Table 5.Y Translation Lookaside Buffer Invalidate Domain Table (TLBI) Format |  |  |  |
| --- | --- | --- | --- |
| Field | Byte Length | Byte Offset | Description |
| Signature | 4 | 0 | “TLBI” Signature for the TLBI Domain Table. |
| Length | 4 | 4 | The length of the table, in bytes, of the entire TLBI. |
| Revision | 1 | 8 | 1 |
| Checksum | 1 | 9 | The entire table, including the checksum field, must add to zero. |
| OEMID | 6 | 10 | An OEM-supplied string that identifies the OEM. |
| OEM Table ID | 8 | 16 | An OEM-supplied string that the OEM uses to identify this particular data table. |
| OEM Revision | 4 | 24 | An OEM-supplied revision number. |
| Creator ID | 4 | 28 | The Vendor ID of the utility that created this table. |
| Creator Revision | 4 | 32 | The revision of the utility that created this table. |
| Number of components (M) | 4 | 36 | The number of TLBI Domain components in the Component list. |
| Component list | -- | 40 | List of M TLBI Domain component sub-tables. The TLBI Domain component sub-table is defined in Table 5.Z |

Table 5.Z  TLBI Domain Component sub-table

| Field | Byte Length | Byte Offset | Description |
| --- | --- | --- | --- |
| Type | 2 | 0 | Component type: 1: PE 2: SMMU All other values are reserved. |
| Reserved | 2 | 2 |  |
| Length | 4 | 4 | Length of this sub-table in bytes, including the Parent domain array. |
| Identifier | 4 | 8 | The identifier of the associated component: If Component type==1: the field contains the “ACPI Processor UID” of the corresponding GICC MADT entry. If Component type==2: the field contains the Identifier number of the corresponding SMMU entry in the IORT. |
| Number of Domains (N) | 4 | 12 | Number of entries in the Domain array. |
| Domain array offset | 4 | 16 | Offset in bytes, from the start of the TLBI table to the start of the Domain array. |
| Domain array[N] | -- | 20 | Array of TLBI Domain IDs, with N entries. This component is contained within every domain ID listed in the array. Each Domain ID is represented by a 32-bit integer. |